PROJECT REPORT

ESP32-Based Vibration Sensor System

with Machine Learning Anomaly Detection

Author: Hymnelaus1

Date: April 2026

Repository: github.com/Hymnelaus1/vibration_project

# 1. Introduction

This project implements a real-time machine health monitoring system using an ESP32 microcontroller equipped with a vibration sensor. Raw analog readings are transmitted over WiFi to a Python-based server, which stores the data, extracts signal features, and applies a One-Class SVM model to detect anomalies. Results are pushed to a live web dashboard via WebSocket.

The system is designed to be low-cost, easy to deploy, and capable of detecting mechanical faults such as bearing damage, imbalance, and looseness without requiring labeled anomaly data during training.

# 2. System Architecture

The pipeline consists of four stages connected in sequence:

| Vibration Sensor (ADC) | → | ESP32 (WiFi POST) | → | Flask Server + ML Model | → | Web Dashboard (WebSocket) |
| --- | --- | --- | --- | --- | --- | --- |

- The vibration sensor outputs an analog voltage signal sampled by the ESP32 ADC.

- ESP32 sends each reading as a JSON payload via HTTP POST over WiFi.

- The Flask server buffers readings in 64-sample windows, extracts features, and runs the SVM model.

- Anomaly flags and raw readings are broadcast to all connected dashboards via WebSocket.

# 3. File Structure & Implementation

## 3.1  server/sunucu.py — Flask REST API Server

The central component of the system. Manages data ingestion, feature computation, ML inference, database storage, and real-time broadcasting.

- POST /data: Receives JSON from ESP32 containing sensor_id, raw ADC value, and voltage.

- A 64-sample sliding window (Python deque) is maintained per sensor ID.

- When the window is full, five signal features are extracted via features.py.

- If a trained model exists (model.pkl), an anomaly score and label are computed.

- All readings are persisted to SQLite (sensor_data.db).

- Results are broadcast to all WebSocket clients via flask_sock.

- GET /readings: Returns the last 60 readings as JSON.

- model.pkl is loaded automatically at startup if it exists.

## 3.2  server/features.py — Signal Feature Extraction

Computes five time-domain and frequency-domain features from each 64-sample voltage window:

| Feature | Formula Basis | What It Detects | Unit |
| --- | --- | --- | --- |
| RMS | sqrt(mean(x²)) | Average vibration energy level | V |
| Peak-to-Peak | max(x) − min(x) | Mechanical looseness | V |
| Kurtosis | E[(x−μ)⁴] / σ⁴ | Bearing damage, shock events | — |
| Crest Factor | max¦x¦ / RMS | Crack and impulse faults | — |
| FFT Energy | Σ¦FFT(x)¦² | Dominant frequency-domain energy | V² |

## 3.3  server/dashboard.html — Real-Time Web Dashboard

A single-page application (SPA) built with plain HTML, CSS, and JavaScript — no framework dependency.

- Chart.js renders a live dual-axis chart: voltage (left axis) and raw ADC / 1000 (right axis).

- On connection, the last 60 historical readings are fetched from the server.

- Each new reading updates the chart and the data table without page reload.

- Connection status is shown as LIVE or RECONNECTING.

- Auto-reconnect logic retries every 3 seconds on disconnect.

## 3.4  ml/train_model.py — One-Class SVM Training

Trains the anomaly detection model using only normal operating data — no anomaly labels required.

- Reads rms, peak_to_peak, kurtosis, crest_factor, fft_energy columns from SQLite.

- StandardScaler normalises the features (essential for SVM: features span very different numeric ranges).

- One-Class SVM is trained with kernel=RBF, nu=0.05, gamma=scale.

- The scaler and model are saved together as a scikit-learn Pipeline in ml/model.pkl.

- nu=0.05 allows up to 5% of training samples to be treated as outliers.

- At inference time, predict() returns -1 for anomaly and +1 for normal.

## 3.5  esp32/ — Firmware Guide

- Documents the expected POST payload format and sensor wiring (GPIO34 analog input).

- Recommended sensor: ADXL335 — analog output, 0–3.3 V, directly compatible with ESP32 ADC.

## 3.6  stm32/ — Embedded Inference (Planned)

- Next phase: convert the trained model to C code using the emlearn library.

- STM32 will handle feature extraction and SVM inference locally on-device.

- ESP32 will only be responsible for WiFi communication and data forwarding.

- This eliminates the need for a server connection during field deployment.

# 4. Technologies Used

| Layer | Technologies Used |
| --- | --- |
| Hardware | ESP32, ADXL335 analog vibration sensor |
| Communication | WiFi, HTTP POST (JSON payload), WebSocket |
| Server | Python, Flask, flask_sock, SQLite |
| Machine Learning | scikit-learn: One-Class SVM, StandardScaler, Pipeline — NumPy, joblib |
| Frontend | HTML5, CSS3, JavaScript, Chart.js, WebSocket API |
| Version Control | Git, GitHub — github.com/Hymnelaus1/vibration_project |

# 5. Setup & Usage

- Install Python dependencies:

pip install flask flask-sock scikit-learn numpy joblib

- Start the server:

python server/sunucu.py

- Power on the ESP32 under normal operating conditions and collect at least 50 data windows.

- Train the model:

python ml/train_model.py

- Restart the server. Anomalies will now be flagged in real time on the dashboard.

# 6. API Reference

| Method | Endpoint | Description |
| --- | --- | --- |
| POST | /data | Receive sensor reading from ESP32 |
| GET | /readings | Return last 60 readings as JSON |
| GET | / | Serve the live web dashboard |
| WS | /ws | WebSocket endpoint for real-time updates |

Example POST body:

{ "sensor_id": "sensor_1", "raw": 2048, "voltage": 1.654 }

# 7. Conclusion & Future Work

The system demonstrates that real-time vibration monitoring and machine learning based anomaly detection can be achieved on low-cost hardware with a minimal software stack. The One-Class SVM approach is particularly suitable for industrial deployments where anomaly labels are unavailable, as it learns a decision boundary purely from normal operating data.

Future work will focus on embedded inference on the STM32 microcontroller using the emlearn library, which converts trained scikit-learn models to C header files. This will enable standalone, server-independent anomaly detection in field conditions. Additional improvements include alert notifications (email / buzzer) and multi-sensor support.

Source code:

https://github.com/Hymnelaus1/vibration_project